DeployedController.__init__() — Loads the Model

This runs automatically when you write ctrl = DeployedController(...). It opens the checkpoint file, reads the saved transformer weights and hyperparameters, rebuilds the exact PITNN architecture that was trained, loads the weights into it, sets dropout to zero since inference does not need it, and restores the normalisation statistics (mu and sigma) that were computed from the training dataset. It also creates a DABPhysics object for physics verification and wraps everything in a PITNNController that manages the rolling 20-step history buffer. After this call the model is fully loaded in memory and ready to predict phase shifts without any further setup.

DeployedController.reset() — Clears the History Buffer

The PITNN is a Transformer that uses the last 20 switching cycles of system state to predict the next phase shift. The reset() call wipes that 20-step buffer back to empty. You call this whenever you are starting a completely new operating scenario — for example, switching from one load condition to a very different one, or when restarting the converter after a fault. If you do not reset between very different conditions, the Transformer will see a history that does not match the current operating point, which causes a brief transient in the predictions until the buffer fills with fresh data.

DeployedController.step() — The Core Inference Call

This is the one function that runs every switching cycle in a real deployment. You give it four numbers from your sensors — V1 (primary bus voltage), V2 (secondary bus voltage), iL (inductor current), and Pref (power reference from the outer control loop) — and it returns the three phase shifts phi1, phi2, phi3.

Inside, it builds the 8-feature state vector by assembling those four measurements with the previous phi values and the voltage product ratio V1·V2/V²nom. It normalises the vector using the training statistics, appends it to the rolling buffer, drops the oldest entry so the buffer stays at exactly 20 steps, then passes the whole buffer through the Transformer in a single forward pass. The output angles are clamped to their valid hardware ranges: phi1 and phi2 to [2.042, 3.110] rad (PHI12_MIN to PHI12_MAX), and phi3 to [0.02, 1.50] rad (PHI_MIN to PHI3_MAX) before being returned.

The optional verify_physics=True argument tells it to also run the DABPhysics simulator on the predicted phase shifts — computing actual power, RMS current, ZVS status, and power error percentage. This is useful for monitoring and logging during commissioning but adds about 2ms of overhead per call, so you disable it in a tight real-time loop once you are confident the system is working correctly.

DeployedController.phi3_to_delay_us() — Unit Conversion

Converts the phi3 output from radians into the time delay in microseconds that you actually program into your gate drive hardware. The conversion is phi3 / (2π × fsw) × 10⁶. For example, phi3 = 0.22 rad at fsw = 100kHz gives a delay of 0.35 µs. This is the number you write to the phase register of your PWM timer or the counter comparison register of your FPGA.

DeployedController.phi1_to_duty_pct() — Unit Conversion

Converts phi1 from radians to duty cycle percentage using phi1 / π × 100. You use this to convert phi1 (predicted by the PITNN each cycle in the range [2.042, 3.110] rad) to primary bridge inner duty cycle percentage. The value varies with load, typically around 70–99%. Apply the same conversion to phi2 for the secondary bridge. On a TI C2000 this maps to the CMPA register of the ePWM module.

DeployedController.export() — Saves Embedded-Ready Files

Saves the trained model in two formats that do not require Python to run, plus the normalisation constants. The TorchScript file (pitnn_scripted.pt) can be loaded directly from C++ using the LibTorch library and runs on NVIDIA Jetson, embedded Linux boards, or any platform that supports the PyTorch C++ API. The ONNX file (pitnn_model.onnx) is the most portable format — it can be converted to TensorRT for GPU acceleration, run through Intel OpenVINO for CPU optimisation, loaded onto ARM Cortex-M microcontrollers via ONNX Runtime for Microcontrollers, or processed by Xilinx Vitis AI for FPGA deployment. The mu and sigma numpy arrays must travel with whichever format you use because the embedded code needs them to normalise the sensor readings before passing them to the model.

OuterPIController.__init__() — Sets Up the Voltage Regulator

Creates the outer proportional-integral controller that generates the Pref power reference from the output voltage error. This corresponds to the paper's Section III, Equations 21 and 22. The parameters you set here are Vref (target output voltage in volts), Kp (proportional gain), Ki (integral gain), Pref_min and Pref_max (power reference limits in watts), and how frequently it updates — defaulting to every 10 switching cycles. The integral accumulator is initialised to zero and an anti-windup clamp is applied to prevent the integral from growing unbounded during startup or fault conditions.

OuterPIController.update() — Computes the Power Reference

Call this every switching cycle with the measured V2. It internally counts cycles and only recalculates Pref every n cycles (defaulting to 10), returning the cached value in between. When it does update, it computes the voltage error as Vref − V2_measured, advances the integral term by error × dt, applies the anti-windup clamp, then computes Pref = Kp × error + Ki × integral and clips it to the [Pref_min, Pref_max] range. The returned Pref is what gets passed to ctrl.step() as the power setpoint for the PITNN.

SafetyMonitor.__init__() — Sets Protection Limits

Initialises the hardware protection layer with overvoltage limits for V1 and V2 (defaulting to 960V, which is 120% of the 800V nominal), an overcurrent limit for iL (defaulting to 200A, above the maximum RMS at 70kW), and the valid phi3 range. A fault counter is also initialised to track how many times the model output has been clamped.

SafetyMonitor.check_sensors() — Validates Sensor Readings

Call this every cycle before passing measurements to the PITNN. It checks all three sensor values against their limits and returns a (safe, reason) tuple. If safe is False, you should immediately disable the gate drive outputs and log the reason string. This catches ADC glitches, sensor failures, or genuine hardware faults before they cause the controller to produce invalid phase shifts.

SafetyMonitor.check_output() — Validates Model Output

Call this after each ctrl.step() call, before applying the phase shifts to the gate drive. It verifies that phi3 is within the valid ascending-branch range [PHI_MIN, PHI3_MAX]. If it falls outside — which should not happen under normal operation since the model output is hard-bounded by the sigmoid — it clamps the value, increments the fault counter, and returns False so your code can log the event. This is a final backstop that ensures no out-of-range value ever reaches the PWM hardware.

run_demo() — Mode 1, Simulated Sensors

Cycles through 20 predefined operating points from 10kW to 60kW and back, adding realistic sensor noise to each measurement, calling ctrl.step() with physics verification enabled, and printing a formatted table showing phi3, the gate delay in microseconds, duty cycle, computed power, ZVS status, and power error percentage. No hardware is needed — this mode runs entirely in software and is the right starting point to confirm the deployment file is working correctly before connecting any hardware.

run_hardware_loop() — Mode 2, Real Hardware

The real-time control loop structure for embedded deployment. It creates an outer PI controller and safety monitor, then loops at the switching frequency calling the sensor stubs, safety checks, outer PI, PITNN inference, output safety clamp, and PWM write in that order. The three lines marked with ← stub are where you replace the simulated values with real ADC reads from your hardware API. The two PWM write lines are where you replace the comments with actual register writes to your gate drive. Every 1000 cycles it prints a status line so you can monitor the system during commissioning. The loop respects the switching period using time.sleep() for the remaining time, though on a proper real-time OS you would use a hardware timer interrupt instead.

run_export() — Mode 3, Embedded File Generation

Calls ctrl.export() and prints a platform-specific guide explaining which exported file to use for each embedded target. Jetson Orin and embedded Linux use the TorchScript file with LibTorch. TensorRT for edge GPU inference uses the ONNX file converted with the trtexec command-line tool. STM32 and ARM Cortex-M microcontrollers use ONNX Runtime for Microcontrollers. TI C2000 DSP requires extracting the weight arrays from the checkpoint and implementing the matrix multiplications directly in C. Xilinx FPGAs use the Vitis AI toolchain with the ONNX file as input.

run_closed_loop() — Mode 4, Full System Simulation

The most complete demonstration mode. It connects the outer PI voltage controller to the PITNN inner modulation layer with the DABPhysics model acting as the plant — a simple capacitor-resistor load model updates V2 each cycle based on the power computed by the physics simulator. This reproduces the full closed-loop control architecture from the paper's Section III and lets you observe how V2 converges toward Vref over time, how the power reference ramps up as the voltage error closes, and whether ZVS is maintained throughout the transient. The printed table every 50 cycles shows Pref, phi3, actual delivered power, V2 voltage, and voltage error, giving a clear picture of the transient and steady-state behaviour before you connect real hardware.

What Each Exported File Contains

pitnn_scripted.pt — The complete trained PITNN frozen as a TorchScript program. It contains the model architecture, all 564,609 trained weights, and the computation graph compiled into a format that runs without pitnn_dab.py being present. Think of it as a self-contained black box: input goes in, phase shifts come out.

pitnn_mu.npy — An array of 8 numbers, one mean value per input feature. Every sensor reading must have its corresponding mean subtracted before being fed into the model, exactly as the training data was normalised.

pitnn_sigma.npy — An array of 8 standard deviations, one per input feature. After subtracting the mean, each feature is divided by its standard deviation. Without this step the model outputs nonsense because it was trained on normalised inputs.

The features in order are: [V1, V2, iL, φ1, φ2, φ3, Pref, V1·V2/V²nom]

pitnn_inspect_exports.py — Run this first. It reads the .npy files and prints the exact normalisation constants formatted for every target language: C++ arrays, Python arrays, MATLAB vectors, and JSON. Also verifies that pitnn_scripted.pt loads and produces valid output. Run with python pitnn_inspect_exports.py.

pitnn_inference.py — Option 1. Pure Python using TorchScript. Put it in the same folder as the three exported files and run python pitnn_inference.py. The PITNNInference class is what you import into your own code — call ctrl.step(V1, V2, iL, Pref) once per switching cycle and get back (phi1, phi2, phi3) in radians. The phi3_to_delay_us() and phi1_to_duty_pct() methods handle unit conversion for your gate drive.

pitnn_onnx_inference.py — Option 3. Uses ONNX Runtime instead of PyTorch, making it compatible with Jetson TensorRT, Intel OpenVINO, Raspberry Pi, STM32, MATLAB, and LabVIEW. Requires pitnn_model.onnx which is generated by running python pitnn_deploy.py --mode export. Includes a benchmark function that measures inference latency over 100 calls and tells you whether it is fast enough for cycle-by-cycle control.

pitnn_inference.cpp — Option 2. C++ implementation using LibTorch. The normalisation constants from your actual .npy files are hard-coded directly into the MU[8] and SIGMA[8] arrays at the top — no runtime file loading required on the target hardware, making this suitable for embedded Linux boards where file I/O is a concern. The PITNNController class has the same step(), reset(), phi3_to_delay_us(), and phi1_to_duty_pct() interface as the Python version.

CMakeLists.txt — Build file for the C++ implementation. Point it at your LibTorch installation and it handles everything including copying the model files next to the executable and, on Windows, copying the LibTorch DLLs.