PITNN DAB Converter PLECS Integration Guide Physics-Informed Transformer Neural Network

Four integration methods — choose the one that matches your PLECS setup

Chukwuemeka Nzeadibe  |  Mississippi State University  |  2026

| Method | File | When to Use | Difficulty |
| --- | --- | --- | --- |
| 1 — C-Script | pitnn_cscript.h | Standalone PLECS, no external process | Medium |
| 2 — Python Co-sim | pitnn_plecs_cosim.py | Easiest setup, standalone PLECS | Easy ★ |
| 3 — MATLAB System Object | PITNNController.m | PLECS Blockset inside Simulink | Medium |
| 4 — Socket Server | pitnn_socket_server.py + pitnn_socket_client.h | PLECS RT / HIL targets | Advanced |

# 1. Project File Structure

Place all files in the same folder as your PLECS simulation:

pitnn_dab\

pitnn_dab.py              ← training + simulation (do not modify)

pitnn_deploy.py           ← deployment controller

pitnn_inference.py        ← standalone Python inference class

pitnn_scripted.pt         ← exported TorchScript model

pitnn_mu.npy              ← normalisation means

pitnn_sigma.npy           ← normalisation std devs

pitnn_model.onnx          ← ONNX model (generate with --mode export)

─── PLECS integration files ───

pitnn_cscript.h           ← Method 1: C-Script declarations

pitnn_plecs_cosim.py      ← Method 2: Python co-simulation

PITNNController.m         ← Method 3: MATLAB System Object

pitnn_socket_server.py    ← Method 4: Socket server (start before PLECS)

pitnn_socket_client.h     ← Method 4: C-Script socket client

─── Your PLECS model ───

DAB_Converter.plecs       ← your PLECS schematic

# 2. Prerequisites

Complete all steps before attempting any integration method:

- Train the model: Run python pitnn_dab.py (or python pitnn_dab.py --video Video_Project.mp4)

- Export the model: Run python pitnn_deploy.py --mode export — creates pitnn_scripted.pt, pitnn_model.onnx, pitnn_mu.npy, pitnn_sigma.npy

- Verify exports: Run python pitnn_inspect_exports.py — confirms all files are valid and prints normalisation constants

- Install dependencies: Run pip install torch numpy scipy opencv-python

Required Files — Must All Exist Before Connecting PLECS

pitnn_scripted.pt — TorchScript model (generated by --mode export)

pitnn_mu.npy — normalisation means

pitnn_sigma.npy — normalisation standard deviations

pitnn_inference.py — inference class (Methods 2, 3, 4 import from this)

# 3. Method 2 — Python Co-Simulation (Recommended Start)

Uses pitnn_plecs_cosim.py. No C code, no socket server. PLECS calls the Python script at each sample step through its built-in Simulation Scripts interface.

## Step 1 — Set the project path

Open pitnn_plecs_cosim.py and update the PITNN_DIR variable at the top:

PITNN_DIR = r"C:\Users\Nzead\Documents\Research\Power electronics control system\Simulation\pitnn_dab"

## Step 2 — Add a Simulation Script in PLECS

- In PLECS, go to: Simulation → Simulation Scripts → Add Script

- Set Language to Python and browse to pitnn_plecs_cosim.py

- Set the Trigger to: Before simulation start + During simulation

## Step 3 — Add the Simulation Script block to your schematic

- Drag a Simulation Script block from the component palette onto your schematic

- Double-click the block → set Input Signals: V1_meas, V2_meas, iL_meas, Pref

- Set Output Signals: phi1, phi2, phi3, delay_us, duty_pct

## Step 4 — Wire the signals

- Inputs: Connect your converter's measured V1, V2, iL signals and your outer PI controller's Pref output to the block inputs

- phi3 output: Connect to the Phase Shift Modulator block's phase input — this is the primary control signal

- phi1/phi2 outputs: Connect to the inner duty cycle inputs of both bridge PWM generators (these are predicted, 2.042–3.110 rad rad = 95% duty)

- delay_us output: Optional — connect to a scope to monitor the gate drive delay in microseconds

## Step 5 — Run

# Do not run pitnn_dab.py or pitnn_deploy.py separately.

# PLECS loads pitnn_plecs_cosim.py automatically when you press Run.

# The first simulation step initialises the PITNN and prints:

#   [PITNN Co-Sim] Controller ready  |  PLECS integration active

# 4. Method 1 — C-Script Block

Uses pitnn_cscript.h. Runs entirely inside PLECS with no external process. Contains the buffer management and normalisation. For the full Transformer forward pass, combine with Method 4 (socket server) or see the comments in the file for embedded weight options.

## Step 1 — Add a C-Script block

- In PLECS, add a C-Script block: Components → Control → C-Script

- Set Inputs (4): V1, V2, iL, Pref

- Set Outputs (5): phi1, phi2, phi3, delay_us, duty_pct

## Step 2 — Open pitnn_cscript.h and copy the three sections

- SECTION 1 → paste into the Declarations tab of the C-Script editor

- SECTION 2 → paste into the Output function tab

- SECTION 3 → paste into the Reset function tab

## Step 3 — Add inference (choose one)

- Option A — Socket server (recommended): Start pitnn_socket_server.py, then replace the placeholder in Section 2 with the socket send/receive calls from pitnn_socket_client.h

- Option B — Python co-simulation: Skip the C-Script inference entirely and use Method 2 instead. Use the C-Script only for the buffer and normalisation logic.

# 5. Method 3 — MATLAB System Object (Simulink + PLECS Blockset)

Uses PITNNController.m. For users running PLECS inside Simulink via the PLECS Blockset.

## Step 1 — Add PITNNController.m to your MATLAB path

addpath('C:\Users\Nzead\...\pitnn_dab')   % in MATLAB Command Window

## Step 2 — Set the project folder

Open PITNNController.m and update the PITNN_DIR property to your project folder path.

## Step 3 — Add to Simulink model

- In Simulink, add a MATLAB System block (from the User-Defined Functions library)

- Double-click → set System object name to: PITNNController

- Wire inputs: V1, V2, iL, Pref from your PLECS Blockset signals

- Wire outputs: phi1, phi2, phi3 to your PLECS PWM generator inputs

## Step 4 — Configure sample time

Set the PLECS Blockset simulation step size to match SampleTime = 1/100e3 (10µs). Both the System Object and the PLECS plant must share the same fundamental sample time.

# 6. Method 4 — Socket Server (PLECS RT / HIL)

Uses pitnn_socket_server.py on the inference PC and pitnn_socket_client.h in the PLECS C-Script block. Best for real-time hardware-in-the-loop targets.

## Step 1 — Start the server before PLECS

# Open a terminal in the pitnn_dab folder

python pitnn_socket_server.py

# Expected output:

# [Server] Listening on 127.0.0.1:9999

# [Server] Waiting for PLECS to connect...

## Step 2 — Configure the server IP

If PLECS and the server run on the same PC, leave HOST = "127.0.0.1". If the server runs on a separate PC connected via LAN, change HOST to that PC's IP address and update PITNN_HOST in pitnn_socket_client.h to match.

## Step 3 — Add C-Script block with socket client

- Add a C-Script block with 4 inputs and 3 outputs

- Open pitnn_socket_client.h and copy Section 1 into Declarations, Section 2 into Output function, Section 3 into Reset function

- Run the simulation — the C-Script connects automatically on the first time step

Server Log During Active Simulation

[Server] PLECS connected from ('127.0.0.1', 54321)

[Server] Calls: 10,000 Avg latency: 1.23µs Last: phi3=0.2198 rad Pref=20000W

[Server] Calls: 20,000 Avg latency: 1.21µs Last: phi3=0.3361 rad Pref=30000W

# 7. Wiring the PITNN Output to Your DAB Schematic

Regardless of which method you use, the outputs connect to your DAB converter schematic the same way:

| PITNN Output | Value | Connect To in PLECS |
| --- | --- | --- |
| phi1 | 2.042–3.110 rad (predicted) | Primary bridge PWM: inner pulse width = phi1/π × Ts/2 |
| phi2 | 2.042–3.110 rad (predicted) | Secondary bridge PWM: inner pulse width = phi2/π × Ts/2 |
| phi3 | 0.05–1.50 rad (varies) | Phase Shift Modulator: external phase delay between bridges |
| delay_us | phi3/(2π×fsw)×10⁶ | Optional scope monitor — gate drive delay in microseconds |
| duty_pct | varies (predicted) | Optional scope monitor — inner duty cycle percentage |

Most important connection: phi3 → Phase Shift Modulator block. This is the only output that changes each cycle. phi1 and phi2 are constant and control the inner duty of both bridges (95% duty = 0.95π rad).

# 8. Outer PI Voltage Controller

The PITNN is the inner modulation layer only. It needs a power reference Pref from an outer voltage controller. Add this structure to your PLECS schematic:

V2_reference ──► [Sum] ──► [PI Controller] ──► Pref ──► PITNN block

V2_measured  ──► [Sum] (negative input)

PI Controller settings (starting values — tune for your plant):

Kp  = 50.0    (proportional gain)

Ki  = 500.0   (integral gain)

Ts  = 100e-6  (PI update every 10 switching cycles)

Min output = 5000 W

Max output = 70000 W

In PLECS, implement this as a Transfer Function block with:

Numerator   = [Kp*Ti + Kp, 0]   where Ti = Kp/Ki

Denominator = [Ti, 0]

Or use a discrete PI block with the Transfer Function block set to z-domain.

# 9. Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| PLECS cannot find pitnn_plecs_cosim.py | PITNN_DIR path is wrong | Update PITNN_DIR in pitnn_plecs_cosim.py to match your folder |
| ModuleNotFoundError: pitnn_inference | pitnn_inference.py not in PITNN_DIR | Check that pitnn_inference.py is in the same folder as pitnn_dab.py |
| FileNotFoundError: pitnn_scripted.pt | Model not exported yet | Run: python pitnn_deploy.py --mode export |
| phi3 output is constant (not changing) | Controller buffer not primed | Add 5-cycle warmup loop or check that Pref signal is connected and non-zero |
| Socket connection refused | Server not running | Start pitnn_socket_server.py before running PLECS simulation |
| C-Script compile error | Missing socket library | On Windows: ensure ws2_32.lib is linked. On Linux: add -lsocket to compile flags |
| phi3 > 1.50 or < 0.05 rad | Clamp not applied | Verify the safety clamp lines are present in Section 2 of your C-Script |
| Simulation very slow | Python overhead per step | Use Method 4 (socket server) — single TCP roundtrip is faster than Python call overhead in co-sim mode |

# 10. Quick Command Reference

# ── BEFORE STARTING PLECS (run once) ────────────────────────────────

python pitnn_dab.py                     # train the model

python pitnn_deploy.py --mode export    # export pitnn_scripted.pt etc.

python pitnn_inspect_exports.py         # verify files + print C arrays

# ── METHOD 2 (Python co-sim) ─────────────────────────────────────────

# No separate command — PLECS calls pitnn_plecs_cosim.py automatically

# ── METHOD 4 (socket server) ─────────────────────────────────────────

python pitnn_socket_server.py           # start BEFORE opening PLECS

# ── VERIFY INFERENCE WORKS INDEPENDENTLY ─────────────────────────────

python pitnn_inference.py               # runs demo without PLECS

python pitnn_onnx_inference.py          # ONNX Runtime version

PITNN DAB Converter — PLECS Integration Guide

Copyright © 2026 Chukwuemeka Nzeadibe

Mississippi State University